Metrologic JavaPOS Driver Manual for Windows V1.2.8

November 6, 2008

1	NOTES.	2

2	Install the Java2 Runtime Environment	2

3	Install the Metrologic Service Object and JavaPos files	2

4	How to use RS232 scanner with Metrologic JavaPOS Driver	2

4.1	Install the Java Communications API	2

4.2	Configuring the Scanner via barcodes	2

4.3	Running the JavaPOS Test utility to evaluate installation	4

4.4	How to Use Metrologic JavaPOS Driver at your application	4

5	How to use POS USB scanner with Metrologic JavaPOS Driver	5

5.1	Configuring the Scanner via barcodes	5

5.2	Install the driver of POS USB	6

5.3	Running the JavaPOS Test utility to evaluate installation	6

5.4	How to Use Metrologic JavaPOS Driver at your application	6

6	How to use IBM OEM USB scanner with Metrologic JavaPOS Driver	7

6.1	Configuring the Scanner via barcodes	7

6.2	Install the JNI library of FSUSB	8

6.3	Running the JavaPOS Test utility to evaluate installation	8

6.4	How to Use Metrologic JavaPOS Driver at your application	8

7	Test the Scanner with the Metrologic Jpos Administrator	9

8	Document Revision Table	10

JavaPOS for Windows is designed for customers who want to connect a scanner running in POS mode to an application written in Java running on a Windows platform.

# NOTES.

- The JRE used to create these directions was “jre-1_5_0_10”.

- Metrologic JavaPOS driver now is compatible with JavaPOS1.7, JavaPOS1.8,  JavaPOS1.9 and JavaPOS1.10

# Install the Java2 Runtime Environment

The JRE is Java’s virtual machine that allows for applications to run on the target computer. The 1.5 version of the JRE is needed to run the JavaPOS application.

(Note: If there is one JRE later than 1.5 has been existed at your system, it’s no need to reinstall one JRE again. So please ignore the following JRE install part.)

- Go to http://java.sun.com, and select this version of Java Runtime Environment.

- Install the program following the instructions.

# Install the Metrologic Service Object and JavaPos files

Go to “\Driver” in release folder. There are two files: mtlgjpos.jar and jpos.xml. Please include mtlgjpos.jar at your CLASSPATH and also copy the related “JPosEntry” option from the jpos.xml to your application’s jpos.xml.

# How to use RS232 scanner with Metrologic JavaPOS Driver

## Install the Java Communications API

Go to Component\JDK118-javaxcomm\windows in release folder. There are three files: win32com.dll, comm.jar,and javax.comm.properties. Please do the following operations to install the library.

1 Copy win32com.dll to C:\Windows\system32.

2 Copy comm.jar to <JRE Install Folder>\lib\ext.

3 Copy javax.comm.properties to <JRE Install Folder>\ib\.

## Configuring the Scanner via barcodes

- Scan the bar codes on the next page, in the sequence given, to configure the scanner.

(Note: if you have a Stratos scanner, use one of the bar codes on page 3, instead of the ones on the other pages.)

JavaPOS configuration bar codes: (scan in order, left column, then right column)

Enable STX Prefix

Enable ETX Suffix

Enable D/E Disable/Enable Commands

Enable LED brightness

If you have a Stratos Scanner/scale, scan ONE of the following bar codes to configure your scanner:

Scanning the following bar code will configure a single-cable Stratos for Metrologic OPOS/JPOS Single Cable RS232 Mode – English.

Scanning the following bar code will configure a single-cable Stratos for Metrologic OPOS/JPOS Single Cable RS232 Mode – Metric.

## Running the JavaPOS Test utility to evaluate installation

If you have a scanner/scale, the scale needs to be entered as a separate device.  It doesn’t matter if it is single cable or dual cable.

Please run the test application at “TestApp” folder in the release folder to have a test of your configuration.

- Test the Scanner with the Metrologic Jpos Administrator.

## How to Use Metrologic JavaPOS Driver at your application

- Go to “\Driver” folder in release folder.

- Include “mtlgjpos.jar” to the CLASSPATH and copy the related “JPosEntry” option from the “jpos.xml”.

- Please select one service object based on your JavaPOS version.

| Class | JavaPos Specification |
| --- | --- |
| com.metrologic.javapos.so.MetroScanRS232v17 | 1.7 |
| com.metrologic.javapos.so.MetroScanRS232v18 | 1.8 |
| com.metrologic.javapos.so.MetroScanRS232v19 | 1.9 |
| com.metrologic.javapos.so.MetroScanRS232v110 | 1.10 |

- Modify the jpos.xml based on your JavaPos version/Scanner model/COM PORT.

- Sample jpos.xml file content

<?xml version="1.0" encoding="UTF-8"?> <!DOCTYPE JposEntries PUBLIC "-//JavaPOS//DTD//EN" "jpos/res/jcl.dtd"> <JposEntries> <!--Saved by JavaPOS jpos.config/loader (JCL) version 2.2.0 on 8/13/07 4:29 PM--> <JposEntry logicalName="MetroScannerRS232"> <creation factoryClass="com.metrologic.javapos.so.MetroScannerServiceFactory" serviceClass="com.metrologic.javapos.so.MetroScanRS232v19"/> <vendor name="Metrologic Instruments, Inc" url="http://www.metrologic.com"/> <jpos category="Scanner" version="1.7"/> <product description="Metrologic bar code scanner" name="Focus/1690" url="http://www.metrologic.com/corporate/products.htm"/> <!--Other non JavaPOS required property (mostly vendor properties and bus specific properties i.e. RS232 )--> <prop name="portName" type="String" value="COM1"/> <prop name="Interface" type="String" value="RS232"/> <prop name="dataBits" type="String" value="8"/> <prop name="Model" type="String" value="Focus/1690"/> <prop name="parity" type="String" value="None"/> <prop name="flowControl" type="String" value="None"/> <prop name="stopBits" type="String" value="1"/> <prop name="deviceBus" type="String" value="RS232"/> <prop name="baudRate" type="String" value="9600"/> </JposEntry> </JposEntries>

# How to use POS USB scanner with Metrologic JavaPOS Driver

## Configuring the Scanner via barcodes

- Scan the bar codes below, in the sequence given, to configure the scanner to POS mode.

JavaPOS configuration bar codes: (scan in order, left column, then right column)

FSUSB

POS USB

Enable STX Prefix

Enable ETX Suffix

Enable D/E Disable/Enable Commands

Enable LED brightness

## Install the driver of POS USB

- Plug the USB cable, the Windows hardware wizard will pop up and help you to install the driver, please install the driver follow the instructions. After install successfully, the system will generate a new virtual serial port.

## Running the JavaPOS Test utility to evaluate installation

- Test the Scanner with the Metrologic Jpos Administrator.

## How to Use Metrologic JavaPOS Driver at your application

- Go to “\Driver” folder in release folder.

- Include “mtlgjpos.jar” to the CLASSPATH and copy the related “JPosEntry” option from the “jpos.xml”.

- Please select one service object based on your JavaPOS version.

| Class | JavaPos Specification |
| --- | --- |
| com.metrologic.javapos.so.MetroScanRS232v17 | 1.7 |
| com.metrologic.javapos.so.MetroScanRS232v18 | 1.8 |
| com.metrologic.javapos.so.MetroScanRS232v19 | 1.9 |
| com.metrologic.javapos.so.MetroScanRS232v110 | 1.10 |

- Modify the jpos.xml based on your JavaPos version/Scanner model/COM PORT.

- Sample jpos.xml file content

<?xml version="1.0" encoding="UTF-8"?> <!DOCTYPE JposEntries PUBLIC "-//JavaPOS//DTD//EN" "jpos/res/jcl.dtd"> <JposEntries> <!--Saved by JavaPOS jpos.config/loader (JCL) version 2.2.0 on 8/13/07 4:29 PM--> <JposEntry logicalName="MetroScannerRS232"> <creation factoryClass="com.metrologic.javapos.so.MetroScannerServiceFactory" serviceClass="com.metrologic.javapos.so.MetroScanRS232v19"/> <vendor name="Metrologic Instruments, Inc" url="http://www.metrologic.com"/> <jpos category="Scanner" version="1.7"/> <product description="Metrologic bar code scanner" name="Focus/1690" url="http://www.metrologic.com/corporate/products.htm"/> <!--Other non JavaPOS required property (mostly vendor properties and bus specific properties i.e. RS232 )--> <prop name="portName" type="String" value="COM6"/> <prop name="Interface" type="String" value="POS USB"/> <prop name="dataBits" type="String" value="8"/> <prop name="Model" type="String" value="Focus/1690"/> <prop name="parity" type="String" value="None"/> <prop name="flowControl" type="String" value="None"/> <prop name="stopBits" type="String" value="1"/> <prop name="deviceBus" type="String" value="RS232"/> <prop name="baudRate" type="String" value="9600"/> </JposEntry> </JposEntries>

# How to use IBM OEM USB scanner with Metrologic JavaPOS Driver

## Configuring the Scanner via barcodes

- Scan the bar codes below, in the sequence given, to configure the scanner to HID mode.

JavaPOS configuration bar codes: (scan in order, left column, then right column)

FSUSB

Enable LED brightness

## Install the JNI library of FSUSB

- Go to “\Component” folder at the release folder.

- Copy the MetroJNIHIDWin.dll to C:\Wondows\System32.

## Running the JavaPOS Test utility to evaluate installation

- Test the Scanner with the Metrologic Jpos Administrator.

## How to Use Metrologic JavaPOS Driver at your application

- Go to “\Driver” folder in release folder.

- Copy “mtlgjpos.jar” and “jpos.xml” to your work folder.

- Go to “\Component” folder at the release folder.

- Copy the MetroJNIHIDWin.dll to C:\Wondows\System32.

- Please select one service object based on your JavaPOS version.

| Class | JavaPos Specification |
| --- | --- |
| com.metrologic.javapos.so.MetroScanFUSBv17 | 1.7 |
| com.metrologic.javapos.so.MetroScanFUSBv18 | 1.8 |
| com.metrologic.javapos.so.MetroScanFUSBv19 | 1.9 |
| com.metrologic.javapos.so.MetroScanFUSBv110 | 1.10 |

- Modify the jpos.xml based on your JavaPos version/Scanner model/COM PORT.

- Sample jpos.xml file content

<?xml version="1.0" encoding="UTF-8"?> <!DOCTYPE JposEntries PUBLIC "-//JavaPOS//DTD//EN" "jpos/res/jcl.dtd"> <JposEntries> <!--Saved by JavaPOS jpos.config/loader (JCL) version 2.2.0 on 8/13/07 4:29 PM--> <JposEntry logicalName="MetroScannerFSUSB"> <creation factoryClass="com.metrologic.javapos.so.MetroScannerServiceFactory" serviceClass="com.metrologic.javapos.so.MetroScanFUSBv19"/> <vendor name="Metrologic Instruments, Inc" url="http://www.metrologic.com"/> <jpos category="Scanner" version="1.7"/> <product description="Metrologic bar code scanner" name="Focus/1690" url="http://www.metrologic.com/corporate/products.htm"/> <!--Other non JavaPOS required property (mostly vendor properties and bus specific properties i.e. RS232 )--> <prop name="hasScale" type="String" value="0"/> <prop name="model" type="String" value="Focus/1690"/> <prop name="interface" type="String" value="Full Speed USB"/> <prop name="productID" type="String" value="0x0009"/> </JposEntry> </JposEntries>

# Test the Scanner with the Metrologic Jpos Administrator

- Run Command Line tool. Go to \TestApp in Release folder. And include the “<JRE_Path>/bin” to the current path.

- Run the MetroJPOStest.bat to start the administrator tool. Be patient, it may take a minute or so for the administrator tool to load.

- Select the New Device tab, and fill in the boxes.

Note that the Profile Name given to the scanner is going to be the same one that you will use for the scanner in administrator tool.

- When you are finished entering the information, click Save. All information you enter will be saved in jpos.xml.

- Select Modify device tab, you can find the profile information you have saved in New Device tab has been listed out. You can modify them, and click Save. Then administrator tool will delete the old information, and save the new.

- Then select Scanner Test tab, enter the profile name you have created in jpos.xml, and click Open, Claim, Enable Data events, Enable Device, Enable Decord data. Now the Administrator Tool is ready to accept data from the scanner. Scan a bar code, and the encoded data will appear in the Administrator window.

If you have a scanner/scale, the scale needs to be entered as a separate device.  It doesn’t matter if it is single cable or dual cable.

# Document Revision Table

| Revision Level | Date | Revised By | Changes Made |
| --- | --- | --- | --- |
| 1.1.0 | 08/08/2005 | M. Lucera | Initial Document written by B. Hejl, S. Defoney and J. Samek was converted to Word format |
| 1.2.0 | 13/08/2007 | Amy | Update the scanner configuration part Driver introduction part and how to run test application. |
| 1.2.7 | 04/047/2008 | John Zhao | (1) Add product id support for IBM OEM USB (2) Modify JNI code for IBM OEM USB for DirectIO support (3) Add Upload/Download/Flash support to DirectIO (4) Add ‘U’ command support to DirectIO (5) Update documents (6) Enable/Disable scanner and adjust LED brightness when enabling/disabling device for RS232 and IBM OEM USB interface. |
| 1.2.8 | 06/11/2008 | SoloYu | - For IBM OEM USB scanner, this version use usage/page to identify it not product id. - Fix memory leak bug at JNI for IBM OEM USB scanner at windows. - Add a prop named DeviceEnable in JposEntry for setDeviceEnable method (4) Change the setDeviceEnable method to set enable/disable device function configurable. |