Metrologic JavaPOS Driver Manual for Linux

V1.2.8

October 27, 2008

1.	NOTES.	2

2.	Linux OS Version.	2

3.	JRE (Java Runtime Environment).	2

4.	The scanner and PC communicate via RS232.	3

4.1	Configure the scanner via barcodes	3

4.2	Java Comm	4

4.3	RXTX Package	4

4.4	Metrologic JavaPOS Driver	5

4.5	Updating the CLASSPATH variables	5

4.6	Running the JavaPOS Test utility	5

5.	The scanner and PC communicate via POS USB.	5

5.1	Configure the scanner via barcodes	5

5.2	Metrologic JavaPOS Driver	7

5.3	Updating the CLASSPATH variables	7

5.4	Running the JavaPOS Test utility	7

2.	Metrologic JavaPOS Driver introduction	7

6.1	RS232	7

6.2	POS USB	8

This JavaPOS for Linux Directions document is for customers who want to connect a scanner to a UPOS application written in Java and running on a Linux platform. The scanner and PC must communicate via RS232, or via POS USB through Metrologic’s POS USB Driver for Linux, in order to use Metrologic’s JavaPOS software on a Linux system.

# NOTES.

  - To install these applications, you will need to have “root” privileges on your system.

  - Linux is case sensitive. Example: “Hello” is different than “hello”

  - The JRE that I used to create these directions was “jre-1_5_0_10”.

  - Metrologic JavaPOS driver now is compatible with JavaPOS1.7, JavaPOS1.8, JavaPOS1.9 and JavaPOS1.10.

  - We assume all the JavaPOS stuff will reside at “root/mtlg/jpos/”.

# Linux OS Version.

These directions were designed and tested on:

  - SUSE 10.2 with kernel 2.6.16.21.

  - SUSE 9 SP3 with kernel 2.6.5.244.

  - Fedora Core 5 with kernel 2.6.15.1

# JRE (Java Runtime Environment).

The JRE is Java’s virtual machine that allows for applications to run on the target computer. The 1.4 version of the JRE is needed to run the JavaPOS application.

Downloading the JRE 1.4 or later version (Java Runtime Environment):

- Download the JRE  from Sun Microsystems:

    - Download at: http://java.sun.com/

    - Select the “Download J2SE JRE” link.

    - On the next page, you will need to “Accept” the licensing agreement.

    - Under the Linux Platform, select the “RPM in self extracting file” to download the JRE.

      - NOTE: an example file name would look like this:

“jre-1_5_0_10-linux-i586-rpm.bin”

Installing the JRE (Java Runtime Environment):

- Create a “JRE” folder on your Linux machine and copy the downloaded JRE file to that folder.

- Open a Terminal window to the “JRE” folder.

- Change the access mode permissions for the JRE file with the following command:

a.	# chmod 755 jre-1_5_0_10-linux-i586-rpm.bin

- The JRE file will then need to be unzipped. The unzipped file will be located in the same directory as the JRE file and will have a RPM extension. After the command is entered, there will be a licensing agreement displayed. You will need to page through the licensing agreement and accept the agreement to continue. To unzip the file enter the following command:

a.	# ./ jre-1_5_0_10-linux-i586-rpm.bin

- After the JRE is unzipped, you will need to install the RPM package. To install the RPM package, enter the following command:

a.	# rpm –ivh jre-1_5_0_10-linux-i586-rpm

- Switch JRE

  - For Fedora6, please modify the /etc/profile and add the following part to the file.

export JAVA_HOME=/usr/java/[jre_version]

export PATH=$JAVA_HOME/bin:$PATH

export CLASSPATH=.:$JAVA_HOME/lib/tools.jar

  - For SUSE

1 .#gedit /etc/profile.d/java.sh，input the following content to the prompted window 
#!/bin/sh

JAVA_HOME=/opt/jdk1.5.0_09/jre
export JAVA_HOME
JAVA_BIN=$JAVA_HOME/bin
CLASSPATH=$CLASSPATH:$JAVA_HOME:$JAVA_HOME/lib
PATH=$JAVA_BIN:$PATH
export JAVA_BIN CLASSPATH PATH

2、source /etc/profile.d/java.sh
#update-alternatives --install /usr/bin/java java /opt/jdk1.5.0_09/jre/bin/java 3

#update-alternatives --config java
please choose 3 at the page

# The scanner and PC communicate via RS232.

## Configure the scanner via barcodes

Enable STX Prefix

Enable ETX Suffix

Enable D/E Disable/Enable Commands

Enable LED brightness

## Java Comm

Java Comm for Linux is a set of free drivers for Sun's official serial driver interface. There is one copy of Comm API at “./component/javaxcomm/linux” folder.

Installing the Java Comm API:

- Copy the “comm.jar” library from “root/mtlg/jpos/component/javaxcomm/linux” to the “/usr/java/[JRE_PATH]/lib/ext” folder.

## RXTX Package

RXTX is a native library providing serial and parallel communication for Java. There is one copy of RXTX at “root/mtlg/jpos/component/javaxcomm/linux” folder.

Installing the RXTX Package:

- Open a Terminal window to the “root/mtlg/jpos/component/javaxcomm/linux” folder.

- The RXTX package file will then need to be unzipped. The RXTX package file will unzip to a folder in the same directory as the file with a folder name of “rxtx-bins.1”. To unzip the file enter the following command:

a.	# tar xvfz rxtx-bins.1.tar.gz

- Open the “rxtx-bins.1” folder that is created from the zip file.

- Copy the “jcl.jar” library from “root/mtlg/jpos/component/javaxcomm/rxtx-bins.1” to the “/usr/java/[jre_folder]/lib/ext” folder.

- In the “rxtx-bins.1” folder, open the subfolder “1.4”, then subfolder “i386-pc-linux” folder.

- Copy the “libSerial.so” and “libSerial-1.4.15.so” files from “/root/java/rxtx-bins.1/1.4/i386-pc-linux” to the “/usr/java/[jre_folder]/lib/i386” folder.

- Create a driver file for the RXTX package. Create a text file and name the file “javax.comm.properties”. In the file, add the text “Driver=gnu.io.RXTXCommDriver”. Save the file to the “/usr/java/j2re1.4.2_13/lib” folder.

## Metrologic JavaPOS Driver

Installing the Metrologic JavaPOS:

- Copy the mtljpos.jar at  .\Driver to your application folder.

## Updating the CLASSPATH variables

- Open the “.bashrc” file in your root (“/root”) directory. The file is a hidden file.

      - NOTE: if the file does not exist on your system. Create a text file named “.bashrc”.

- Add the following text to the bottom of the “.bashrc” file. The following text must be typed just as it appears with the only space being between the word “export” and “CLASSPATH…”:

a. export CLASSPATH=$CLASSPATH:.:/usr/java/[JRE_PATH]/lib/ext/comm.jar:/usr/java/[JRE_PATH]/lib/ext/jcl.jar:/root/java/jpos110.jar:/root/java/mtlgjpos.jar:/root/java/xerces.jar

## Running the JavaPOS Test utility

We provide one test tool for the JavaPOS driver, which is located at . /TestApp

# The scanner and PC communicate via POS USB.

## Configure the scanner via barcodes

FSUSB

POS USB

Enable STX Prefix

Enable ETX Suffix

Enable D/E Disable/Enable Commands

Enable LED brightness

## Metrologic JavaPOS Driver

Installing the Metrologic JavaPOS:

- Copy the mtljpos.jar at  ./Driver to your application folder.

- Copy the libPOSUSBOper.so at ./component folder to you application folder.

## Updating the CLASSPATH variables

      - Open the “.bashrc” file in your root (“/root”) directory. The file is a hidden file.

NOTE: if the file does not exist on your system. Create a text file named “.bashrc”.

      - Add the following text to the bottom of the “.bashrc” file. The following text must be typed just as it appears with the only space being between the word “export” and “CLASSPATH…”:

export CLASSPATH=$CLASSPATH:.:/root/java/jpos110.jar:/root/java/mtlgjpos.jar:/root/java/xerces.jar

## Running the JavaPOS Test utility

We provide one test tool for the JavaPOS driver, which is located at ./TestApp/

# Metrologic JavaPOS Driver introduction

## RS232

| Class | JavaPos Specification |
| --- | --- |
| com.metrologic.javapos.so.MetroScanRS232v17 | 1.7 |
| com.metrologic.javapos.so.MetroScanRS232v18 | 1.8 |
| com.metrologic.javapos.so.MetroScanRS232v19 | 1.9 |
| com.metrologic.javapos.so.MetroScanRS232v110 | 1.10 |

For all the versions, we must support a logicalName and some properties in jpos.xml file.

Please see sample for MetroScanRS232v18 below:

<JposEntry logicalName="MetroScanner2"> <creation factoryClass="com.metrologic.javapos.so.MetroScannerServiceFactory" serviceClass="com.metrologic.javapos.so.MetroScanRS232v18"/> <vendor name="Metrologic Instruments, Inc" url="http://www.metrologic.com"/> <jpos category="Scanner" version="1.8"/> <product description="Metrologic bar code scanner" name="MS5145" url="http://www.metrologic.com/corporate/products.htm"/> <!--Other non JavaPOS required property (mostly vendor properties and bus specific properties i.e. RS232 )--> <prop name="portName" type="String" value="COM1"/> <prop name="dataBits" type="String" value="8"/> <prop name="HasScale" type="String" value="0"/> <prop name="Model" type="String" value="MS5145"/> <prop name="parity" type="String" value="None"/> <prop name="flowControl" type="String" value="None"/> <prop name="stopBits" type="String" value="1"/> <prop name="deviceBus" type="String" value="RS232"/> <prop name="baudRate" type="String" value="9600"/> </JposEntry>

## POS USB

| Class | JavaPos Specification |
| --- | --- |
| com.metrologic.javapos.so.MetroScanPOSUSBForLinux17 | 1.7 |
| com.metrologic.javapos.so.MetroScanPOSUSBForLinux18 | 1.8 |
| com.metrologic.javapos.so.MetroScanPOSUSBForLinux19 | 1.9 |
| com.metrologic.javapos.so.MetroScanPOSUSBForLinux110 | 1.10 |

The sample configuration posentry is as following.

<JposEntry logicalName="MetroScanner2"> <creation factoryClass="com.metrologic.javapos.so.MetroScannerServiceFactory" serviceClass="com.metrologic.javapos.so. MetroScanPOSUSBForLinux18"/> <vendor name="Metrologic Instruments, Inc" url="http://www.metrologic.com"/> <jpos category="Scanner" version="1.8"/> <product description="Metrologic bar code scanner" name="MS5145" url="http://www.metrologic.com/corporate/products.htm"/> <prop name="productID" type="String" value="0x0002"/> <prop name="Model" type="String" value="MS5145"/> <prop name="Debug" type="String" value="true"/> --This is the debug switch </JposEntry>

Document Revision Table:

| Revision Level | Date | Revised By | Changes Made |
| --- | --- | --- | --- |
| 1.0.0 | 08/08/2005 | M. Lucera | Initial Document written by B. Hejl, S. Defoney and J. Samek was converted to Word format |
| 1.2.0 | 08/10/2007 | M.John | Update the JRE install part and add the JSR-80 install part and how to run test application. |
| 1.2.5 | 9/21/2007 | M.John | Add debug switch for the POS USB |
| 1.2.6 | 9/24/2007 | M.John,Amy | Modify the TestApp interface. Modify the POS USB driver’s communication part code |
| 1.2.7 | 4/10/2008 | M.John | (1) Remove JSR driver and adopt JNI for POS USB interface at SUSE9 (2) Add ‘U’ commands support to DirectIO (3) Add “Upload/Download/Flash” support to DirectIO (4) Enable/Disable scanner and adjust LED brightness when enabling/disabling device for RS232 interface |
| 1.2.9 | 6/12/2008 | M.John | Add RemoteMasterMND support to JavaPOS driver |

# Trouble Shot

| Problem | Solution |
| --- | --- |
| When open one serial port and not close/release it following the right way, there will be problem if you reopen it. It will show ‘/var/spool/locks/L…LOCKS’. | Delete the file ‘/var/spool/locks/L…LOCKS’ |